Tutorial 2 : HPLC Simulations

In this tutorial, we use an HPLC simulator developed by the Group of analytical sciences, School of pharmaceutical sciences, University of Geneva. Download the simulator, (Practical HPLC simulator v1.0) in the form of an Excel file, from the address:

https://ispso.unige.ch/labs/fanal/practical_hplc_simulator

1. Analysis of a mixture of parabens (mix. 1)

1.1 Physicochemical properties of the compounds

Methylparaben 	Ethyl paraben	 	Propylparaben		Butylparaben

logP=1,67 		logP=2,03		log P=2,55	   	logP=3,00

pKa=8,5		pKa=8,5		pKa=8,5	   	pKa=8,5

1.2. Elution order.

Consider the following operating conditions:

Column: Waters Acquity BEH C18 (150x4,6 mm, 5m).

Eluent : Acetonitrile /phosphate buffer pH=7: 40/60 v/v

Flow rate : 1mL/min

Temperature : 30°C

Detector: 220nm

Predict the order of elution:

First eluted:	___________________________

___________________________

___________________________

Last eluted	___________________________

Open HPLC simulator. Introduce the above operating conditions to study mixture 1. Compare the obtained order of elution with the predicted one and explain the differences.

1.3. Dead time

Dead time can be calculated (see tutorial 1) or evaluated by injection of unretained compounds. In this simulation, dead time is evaluated using uracil.

.

logP=-0,86,  pKa=8,8

Explain why this compound is unretained in these chromatographic conditions.

1.4. Simulations

1.4.1. Wavelength choice

Optimize the UV wavelength, using the spectra window and the detector window.

1.4.2. Mobile phase:

Modify the mobile phase composition:

-Solvent strength (% organic solvent; polarity)

-Solvent type (comparison of acetonitrile and methanol strength and selectivity)

-pH

Describe the impact on:

- Dead time

- Retention time

- Retention factor (LSS model)

- Separation (resolution)

- Back pressure

1.3.3. Flow rate

Introduce the follow values of flow rate: 0,5mL/min, 1mL/min, 1,5mL/min, 2mL/min. Describe the impact on:

- Dead time

- Retention time

- Retention factor

- Separation (critical resolution)

- Back pressure (plot: pressure = f (flow rate)

1.3.4. Column

-column length

-particle size

-phase type

Describe the impact on:

- Dead time

- Retention time

- Retention factor

- Separation (resolution)

- Back pressure (plot: pressure = f(L))

- Column efficiency (N)

  - Optimisation

Choose the conditions to obtain:

- 1<k<10 and

- R>1,5

- time<15min.

2. Analysis of a mixture of acids (NAIS Mix.2)

2.1 Physicochemical properties of the compounds

Acetylsalicylic acid		paracetamol		 salicylic acid		ketoprofen

logP=1,24			logP=0,91		logP=1,98		logP=3,61

pKa=3,41			pKa=9,46		pKa=2,79		pKa=3,88

Ibuprofen				mefenamic acid

logP=3,84				logP=5,4

pKa=4,85				pka=3,89

2.2. Elution order.

Predict the order of elution:

First eluted:	___________________________

___________________________

___________________________

___________________________

___________________________

Last eluted	___________________________

Introduce the following operating conditions to the simulator and study the mixture 2:

Column: Waters Acquity BEH C18 (150x4,6 mm, 5m).

Eluent : Acetonitrile /phosphate buffer pH=7: 30/70 v/v

Flow rate : 1mL/min

Temperature : 30°C

Detector: 220nm

Compare the obtained order of elution with the predicted one and explain the differences.

2.2. Simulations.

2. 2.1. Separation of the mixture

Find the conditions to obtain acceptable resolution values by only modifying organic solvent (B) content and pH.

Explain why gradient elution is mandatory.

2.2.2. Optimize the conditions of gradient elution

Find gradient conditions to obtain: R>1,5 and time needed for analysis <15 min.

Simulation Report

Student Name 1:							Date:

Student Name 2:

1. Analysis of a mixture of parabens (mix. 1)

Question 1: Predict the elution order of parabens and compare with the obtained chromatogram. Why dead time is evaluated by uracil?

Question 2: Explain how you choose the optimum wavelength.

Question 3: Describe how mobile phase composition in RP-HPLC affect retention and separation: `

|  | Acetonitrile 50% pH=7 | Acetonitrile 40% pH=7 | Acetonitrile 30% pH=7 | MeOH 40% pH=7 | Acetonitrile 40% pH=9 |
| --- | --- | --- | --- | --- | --- |
| Dead time (min) |  |  |  |  |  |
| tR butylparaben (min) |  |  |  |  |  |
| k butylparaben |  |  |  |  |  |
| Critical Resolution |  |  |  |  |  |
| Pressure (bar) |  |  |  |  |  |

Question 4: Describe how flow rate affect retention and separation: `

|  | D=0,5mL/min | D=1mL/min | D=1,5mL/min | D=2mL/min |
| --- | --- | --- | --- | --- |
| Dead time (min) |  |  |  |  |
| tR butylparaben (min) |  |  |  |  |
| k butylparaben |  |  |  |  |
| Critical Resolution |  |  |  |  |
| Pressure (bar) |  |  |  |  |

Question 5: Describe how stationary phase type RP-HPLC and column dimensions affect retention and separation: `

|  | C18 L=150mm dp=5mm | C18 L=50mm dp=5mm | C18 L=150mm dp=3mm | C4 L=150mm dp=5mm | Phenyl L=150mm dp=5mm |
| --- | --- | --- | --- | --- | --- |
| Dead time (min) |  |  |  |  |  |
| tR butylparaben (min) |  |  |  |  |  |
| k butylparaben |  |  |  |  |  |
| Critical Resolution |  |  |  |  |  |
| Pressure (bar) |  |  |  |  |  |
| Efficiency propyl paraben(N) |  |  |  |  |  |

Question 6: Propose optimum operating conditions.

Insert Optimized chromatogram (* paste special image)

2. Analysis of a mixture of acids (NAIS Mix.2)

Question 7: Modify mobile phase composition to find isocratic conditions that separate all compounds.

Question 8: Predict the elution order and compare with the obtained chromatogram (with acceptable resolution values).

Question 9: Explain why gradient elution is preferred.

Question 10: Optimize gradient elution.

Insert optimized chromatogram (*paste special image)